SECTION 07 05 43.05

STRUCTURAL THERMAL FRAMING FOR WINDOWS AND OPENINGS

GreenGirt XO™ Composite Metal Hybrid Fenestration Support System — Specification Short Form

Document: A2P-E-XO-SPC02-v2.4 • 2026-05-21

Owner: Engineering

PART 1: GENERAL

1.01–1.04: Administrative

Refer to A2P-E-XO-SPC01 (Spec Long Form) for complete Section Includes, Related Sections, Reference Standards, and Submittal requirements.

1.05 Performance Requirements — ALL SEVEN (7) REQUIRED

Systems shall meet ALL of the following. Systems that meet only partial requirements shall not be considered acceptable substitutes. All criteria must be demonstrated through independent third-party verification or PE-stamped documentation.

| # | Requirement | Threshold | Verification |
| --- | --- | --- | --- |
| 1 | Thermal Performance | Psi ≤ 0.025 Btu/(hr·ft·°F) [0.043 W/(m·K)] at sill, head, and jamb; wall assembly context disclosed | Third-party per ISO 10211 |
| 2 | Thermal Plane Alignment | Outermost glazing flush with or recessed up to 1” inboard of CI exterior face; shall not extend outboard | Section detail + field verification |
| 3 | Structural Performance | Project-specific PE-stamped calcs; eccentric cantilever analysis; anisotropic properties of composite, lumber, and non-metal materials accounted for; point loading at fastener locations representative of actual cladding dynamics; thermal cycling effects on connections; all fasteners into metal backers or bolted connections | PE-stamped calcs for project loads |
| 4 | Air/Water Resistance | 50 psf min. per ASTM E2357 (air) and ASTM E331 (water); air leakage ≤ 0.04 cfm/ft² at 75 Pa; max water absorption ≤ 0.5%; non-porous, non-absorptive material; no surface sealing of member faces required | Third-party test reports |
| 5 | Fire Safety | Non-combustible dead load support per IBC; ASTM E84 Class A (FSI ≤25, SDI ≤450); NFPA 285 where required | Test reports or letter of equivalency |
| 6 | Single-Source Interface System | Modular aluminum interface flanges via factory keyway; all components from one manufacturer, one warranty; compatible with manufacturer’s CI sub-framing system | Warranty documentation |
| 7 | Certified Assembly Thermal Performance | Assembly Thermal Efficiency ≥ 93%; chi ≤ 0.005 Btu/(hr·°F) per bracket; thermal flanking analysis; 12” simulation boundary in all directions | Independent third-party certification per ISO 10211 with project-state PE stamp |

PART 2: PRODUCTS (BASIS OF DESIGN)

Manufacturer: Advanced Architectural Products (A2P)  |  www.GreenGirt.com  |  (269) 355-1818

Product: GreenGirt XO™ Structural Thermal Framing  |  CMH Technology  |  CSI 07 05 43.05

Available Depths: 2”, 3”, 4”, 5”, 6”

System Components

A.	CMH framing members (sill, jamb, header)

B.	Modular aluminum interface flanges (mini flange, flange, header angle; standard and spacer variants)

C.	Attachment clips (dead load + lateral; aluminum and galvanized)

D.	Mechanical fasteners (per substrate)

E.	Self-sealing anchor plates

Product Performance Summary

| Category | Product Data |
| --- | --- |
| Thermal (4” depth) | Psi 0.0208 Btu/(hr·ft·°F) [0.036 W/(m·K)] per RDH/ISO 10211 |
| Structural | Up to 300 plf dead load at outer edge |
| Air Infiltration | Tested and passed at 100 psf per ASTM E2357 |
| Water Penetration | Tested and passed at 100 psf per ASTM E331 |
| Fire | FSI ≤ 25, SDI ≤ 450 per ASTM E84; NFPA 285 assembly tested |
| Material | Non-porous, non-absorptive; no surface sealing required |
| Assembly Thermal Efficiency | 93–97% across all depths; third-party certified per ISO 10211 |

PART 3: EXECUTION

Installation Sequence: Dead load clips → Interface flanges to CMH (if not pre-assembled) → Sill → Jambs → Header → Seal all corner joints. Install per manufacturer’s instructions and approved shop drawings.

AWB Integration: CMH material is non-porous — AWB ties into CMH flanges at interface only, not on member faces. Do not wrap or coat CMH surfaces.

Coordination: CI installs over/around CMH. Glazier mounts fenestration to CMH members.

SUBMITTALS REQUIRED

A.	Product data sheets

B.	Third-party Psi report per ISO 10211

C.	ASTM E84 test report (FSI/SDI)

D.	NFPA 285 report or letter of equivalency (where required)

E.	ASTM E2357 at 50 psf min. (air infiltration)

F.	ASTM E331 at 50 psf min. (water penetration)

G.	Material water absorption data (ASTM D570)

H.	PE-stamped structural calculations (eccentric cantilever)

I.	Single-source warranty documentation

J.	Shop drawings (layouts, clips, interface flanges, AWB details)

K.	AWB compatibility documentation

L.	Joint sealant specification

M.	Installation instructions

N.	Certified Assembly Thermal Performance report per PR7 (assembly boundary, efficiency calculation, chi values, thermal flanking analysis, isothermal plots, independent PE certification)

SUBSTITUTION NOTICE: Substitution requests shall demonstrate compliance with ALL seven (7) performance requirements in Section 1.05. Partial compliance is not acceptable. Refer to Section 01 60 00. For complete specification including stainless steel alternate, refer to A2P-E-XO-SPC01 (Spec Long Form).

END OF SECTION 07 05 43.05 — SHORT FORM