GREENGIRT XO™ TRADE SCOPE MATRIX

Typical Trade Responsibilities for Bid Package Coordination

Document: A2P-O-XO-MTX01-v1.1 • 2026-02-23     Section: 07 05 43.05: Structural Thermal Framing

GreenGirt XO minimum product scope = 5 items.  All other trades typically continue their normal work. GreenGirt XO is not a new trade — it's a new product in an existing workflow.

RESPONSIBILITY MATRIX

| Scope Item | Typical Description | GreenGirt XO Installer | Carpentry | AWB Installer | Insulation Installer | Glazier | Sealant Installer | Cladding Installer | Drywall Installer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMH framing members | Supply and install GreenGirt XO framing at rough openings | ✔ |  |  |  |  |  |  |  |
| Aluminum interface flanges | Install to CMH members via keyway (if not pre-assembled) | ✔ |  |  |  |  |  |  |  |
| Attachment clips | Clips securing CMH members to structure | ✔ |  |  |  |  |  |  |  |
| Fasteners (CMH to structure) | Mechanical fasteners at clip locations | ✔ |  |  |  |  |  |  |  |
| Sealant at CMH joints | Sealant at CMH-to-CMH corner connections | ✔ |  |  |  |  |  |  |  |
| Rough opening framing | Wood or steel framing of window/door openings |  | ✔ |  |  |  |  |  |  |
| Air/water barrier | AWB membrane application; ties into CMH flanges at interface (not on member faces) |  |  | ✔ |  |  |  |  |  |
| Window opening drip pan | Sill pan flashing at rough opening; ties into WRB system |  |  | ✔ |  |  |  |  |  |
| Interior perimeter air seal | Tape or sealant at interior window-to-wall interface |  |  | ✔ |  |  |  |  |  |
| Continuous insulation | CI board installation over and around CMH framing |  |  |  | ✔ |  |  |  |  |
| Window/door installation | Fenestration units mount to CMH framing (instead of steel angle) |  |  |  |  | ✔ |  |  |  |
| Perimeter sealant | Standard window/door perimeter sealant |  |  |  |  |  | ✔ |  |  |
| Exterior cladding | Cladding system installation per typical practice |  |  |  |  |  |  | ✔ |  |
| Interior finishing | Drywall and interior trim per typical practice |  |  |  |  |  |  |  | ✔ |

Shaded rows = XO product installer scope     White rows = Existing trades, typical scope (no change from standard practice)

NOTES

1.  This matrix reflects typical trade responsibilities and is provided as a coordination reference. Actual scope assignments may vary by project and should be confirmed in bid documents.

2.  The GreenGirt XO installer is typically the same trade that frames rough openings or a specialty subcontractor. Installation training and field support are available from A2P.

3.  AWB integration note: CMH material is inherently non-porous. AWB tie-in occurs at the flange-to-membrane interface only — not on CMH member faces. Do not wrap or coat CMH surfaces. Communicate this to the AWB contractor to prevent unnecessary material use and potential compatibility issues.

4.  The window opening drip pan and interior perimeter air seal are shown under the AWB contractor, as these items maintain air/water barrier continuity at the rough opening. In some markets or project configurations, the glazier may carry sill pan scope — confirm in bid documents.

5.  The glazier's scope is unchanged — windows mount to CMH framing in the same manner they would mount to a traditional steel angle.

6.  For detailed installation sequencing, refer to the Installer Notes (O-XO-MAN01).

Questions? Advanced Architectural Products (A2P) | Manufacturer of a portfolio of continuous insulation and building enclosure systems | (269) 355-1818 | www.GreenGirt.com

GreenGirt CMH XO™ — Trade Scope Matrix • A2P-O-XO-MTX01-v1.1 • 2026-02-23