GREENGIRT XO™

SUBSTITUTION EVALUATION FORM

Section 07 05 43.05: Structural Thermal Framing for Windows and Openings

Document: A2P-E-XO-FRM01-v1.2 • 2026-05-21

Owner: Engineering

INSTRUCTIONS: Complete one form for each substitution request received. Check each box only when documentation has been provided and verified. File the completed form in the project record. If ANY box is unchecked, the substitution does not meet specification. See the Architect Defense Guide (E-XO-GDE01) for detailed evaluation rationale and building science context.

PROJECT INFORMATION

Project Name:  ________________________________________________

Project Number:  ________________________   Date:  ________________________

Specified Product:  GreenGirt XO™ per Section 07 05 43.05

Proposed Substitute:  ____________________________________________

Proposed Manufacturer:  __________________________________________

Submitted By:  ________________________   Submittal Date:  __________________

Evaluated By:  ________________________   Evaluation Date:  __________________

PERFORMANCE REQUIREMENT EVALUATION

ALL SEVEN (7) performance requirements must be met. Partial compliance is not acceptable.

PR1: THERMAL PERFORMANCE

☐  Third-party Psi values provided (not R-values)

☐  Psi ≤ 0.025 Btu/(hr·ft·°F) [0.043 W/(m·K)] at sill, head, AND jamb

☐  Simulation per ISO 10211 methodology

☐  Documentation from independent testing organization (not manufacturer in-house)

☐  Wall assembly context disclosed (backup framing, CI depth, fenestration type)

PR1 Result: ☐ PASS  ☐ FAIL   Notes: _________________________________________________

PR2: THERMAL PLANE ALIGNMENT

☐  Section detail provided showing glazing alignment with CI plane (dimension annotated)

☐  Outermost glazing flush with or recessed up to 1” inboard of CI exterior face (not outboard)

☐  Thermal flanking eliminated at all perimeter conditions

PR2 Result: ☐ PASS  ☐ FAIL   Notes: _________________________________________________

PR3: STRUCTURAL PERFORMANCE

☐  PE-stamped calculations for project-specific loading

☐  Eccentric cantilever loading analysis included

☐  Anisotropic material properties accounted for (BOTH lengthwise AND crosswise values provided)

☐  All fasteners terminate in metal backers or bolted connections only (not composite/FRP/wood)

PR3 Result: ☐ PASS  ☐ FAIL   Notes: _________________________________________________

PR4: AIR, WATER, AND VAPOR RESISTANCE

☐  Air infiltration test report at 50 psf minimum per ASTM E2357

☐  Water penetration test report at 50 psf minimum per ASTM E331

☐  Test pressure is NOT lower than specified (not 12, 20, or 25 psf)

☐  Framing material is non-porous and non-absorptive (ASTM D570 data provided)

☐  Reports from independent, accredited third-party laboratory

PR4 Result: ☐ PASS  ☐ FAIL   Notes: _________________________________________________

PR5: FIRE SAFETY

☐  Dead load support members are non-combustible per IBC Section 202

☐  ASTM E84 Class A (FSI ≤ 25, SDI ≤ 450) documented

☐  NFPA 285 assembly test report or letter of equivalency from fire protection engineer (where required by IBC 1403.5)

PR5 Result: ☐ PASS  ☐ FAIL   Notes: _________________________________________________

PR6: SINGLE-SOURCE MODULAR INTERFACE SYSTEM

☐  All system components (framing, interface hardware, clips, accessories) from same manufacturer

☐  Single-source warranty documentation provided covering all components

☐  Interface components are factory-engineered (not field-fabricated)

PR6 Result: ☐ PASS  ☐ FAIL   Notes: _________________________________________________

PR7: CERTIFIED ASSEMBLY THERMAL PERFORMANCE

☐  Complete assembly boundary defined (all framing members, brackets, fasteners, flanges, insulation, flashing)

☐  Assembly Thermal Efficiency ≥ 93%

☐  OR alternative compliance: Psi meeting PR1 threshold at sill/head/jamb AND chi ≤ 0.005 Btu/(hr·°F) per bracket — both per ISO 10211

☐  Chi-value at each bracket/clip location documented

☐  Thermal flanking paths identified and mitigated

☐  Simulation boundary extends minimum 12” beyond assembly in all directions

☐  Backup wall framing included in simulation (product NOT modeled in isolation)

☐  Isothermal plots and/or thermal flux diagrams provided

☐  Independent third-party consultant performed simulation (not manufacturer in-house)

☐  ISO 10211 compliance statement included in report

☐  Approval and stamp by a Professional Engineer (PE) licensed in the project state

☐  Certification is project-specific (actual CI depth, framing type, bracket spacing)

PR7 Result: ☐ PASS  ☐ FAIL   Notes: _________________________________________________

DOWNSTREAM IMPACT VERIFICATION

☐  Energy model re-run with proposed product’s actual Psi values; building still meets energy code

☐  Mechanical load calculations updated; HVAC equipment confirmed adequate for revised thermal loads

☐  Responsible party identified for any increased lifecycle operating costs

☐  Project schedule impact of re-analysis assessed (typically 4–8 weeks)

EVALUATION SUMMARY

| PR | Requirement | Result |
| --- | --- | --- |
| 1 | Thermal Performance | ☐ PASS ☐ FAIL |
| 2 | Thermal Plane Alignment | ☐ PASS ☐ FAIL |
| 3 | Structural Performance | ☐ PASS ☐ FAIL |
| 4 | Air/Water Resistance | ☐ PASS ☐ FAIL |
| 5 | Fire Safety | ☐ PASS ☐ FAIL |
| 6 | Single-Source Interface System | ☐ PASS ☐ FAIL |
| 7 | Certified Assembly Thermal Performance | ☐ PASS ☐ FAIL |
| — | Downstream Impact Verification | ☐ PASS ☐ FAIL |

☐  SUBSTITUTION APPROVED

All seven performance requirements and downstream impacts verified. Product meets specification.

☐  SUBSTITUTION REJECTED

One or more performance requirements not met. Use applicable rejection language below.

Failed Requirements:  ____________________________________________

REJECTION LANGUAGE — SELECT AND CUSTOMIZE

General (Incomplete Documentation):

“The proposed substitution does not include documentation demonstrating compliance with all seven performance requirements specified in Section 1.05. Substitution rejected. Provide the specified product or a compliant alternative with complete documentation per the specification.”

Partial Compliance:

“The proposed product demonstrates compliance with [X] of seven requirements but does not meet [specific requirements]. The specification requires compliance with all seven interdependent performance criteria. Systems meeting only partial requirements shall not be considered acceptable substitutes.”

Inferior Thermal Performance (PR1):

“The proposed product’s published Psi values do not demonstrate compliance with the ≤ 0.025 Btu/(hr·ft·°F) threshold in the wall assembly context that will be built. Accepting this substitution would degrade the thermal performance on which the building’s energy model, HVAC sizing, and code compliance are based. Substitution rejected.”

Inadequate Test Pressure (PR4):

“The submitted test reports show air/water testing at [12/20] psf. The specification requires 50 psf per ASTM E2357 and ASTM E331. The basis-of-design product has been tested at 100 psf. Testing at lower pressures does not demonstrate equivalent performance. Substitution rejected.”

Absorptive Material (PR4):

“The proposed framing material does not meet the non-porous, non-absorptive requirement. Materials that absorb moisture, swell, or change dimension are not acceptable for fenestration perimeter framing per Section 1.05.D. Substitution rejected.”

Incomplete Assembly Certification (PR7):

“The proposed product has not provided a Certified Assembly Thermal Performance report per Performance Requirement 7. The submittal includes only material-level thermal data without complete assembly boundary analysis, thermal flanking assessment, chi-value documentation at brackets, or independent third-party certification. Material-level data alone does not demonstrate installed assembly performance. Substitution rejected.”

SIGNATURES

Evaluator:  ____________________________   Title:  ____________________________

Signature:  ____________________________   Date:  ____________________________

Architect of Record (if different):  ____________________   Date:  __________________

For evaluation support, contact A2P Engineering: (269) 355-1818  |  engineering@greengirt.com  |  www.GreenGirt.com

For detailed building science rationale behind each performance requirement, refer to the Architect Defense Guide (E-XO-GDE01).

GreenGirt XO™ Substitution Evaluation Form  •  A2P-E-XO-FRM01-v1.2  •  2026-05-21