GREENGIRT XO™ SPECIFICATION DEFENSE

QUICK REFERENCE: 7 Questions to Ask Any Proposed Substitute

Document: A2P-S-XO-GDE01-v2.3 • 2026-05-21

Owner: Sales

Your specification includes seven interdependent performance requirements for fenestration support framing. These requirements will likely be challenged during bidding. When a substitution request arrives, ask these seven questions before evaluating the product:

| 1 THERMAL | Ask: “Provide third-party Psi values for sill, head, and jamb — not R-values.” Requirement: Psi ≤ 0.025 Btu/(hr·ft·°F) [0.043 W/(m·K)] per ISO 10211. Red flag: R-value claims without Psi data. Psi data without identification of the wall assembly context (backup framing, CI depth, fenestration type) modeled. |
| --- | --- |
| 2 ALIGNMENT | Ask: “Show section detail demonstrating glazing alignment with the exterior face of the CI plane.” Requirement: Outermost glazing surface flush with or recessed up to 1” inboard of CI plane, shall not extend outboard. Red flag: No section detail provided. |
| 3 STRUCTURAL | Ask: “Provide PE-stamped calculations for this project’s loading, including eccentric cantilever analysis. Confirm all fasteners terminate in metal backers or bolted connections.” Requirement: Project-specific PE-stamped calcs. Anisotropic properties of composite, lumber, or non-metal materials accounted for. All fasteners into metal backers or bolted connections only. Red flag: Uniform load calcs only; no PE; fasteners into composite or FRP without metal reinforcement. |
| 4 AIR/WATER | Ask: “What pressure was testing performed at? Is the framing material non-porous?” Requirement: 50 psf per ASTM E2357 (air) and ASTM E331 (water). Non-absorptive material. Red flag: Testing at 12 or 20 psf. Material requires surface wrapping. |
| 5 FIRE SAFETY | Ask: “Are dead load support members non-combustible per IBC? Provide ASTM E84 and NFPA 285 documentation.” Requirement: Non-combustible per IBC Section 202. ASTM E84 Class A. NFPA 285 where required. Red flag: Plastic-only load-bearing components. |
| 6 SINGLE-SOURCE | Ask: “Are all components — framing, interface hardware, clips — from one manufacturer under one warranty?” Requirement: Factory-engineered modular interface flanges. Single manufacturer, single warranty. Red flag: Interface components “by others” or split warranty. |
| 7 ASSEMBLY | Ask: “Provide the complete assembly thermal certification — including all brackets, flanking paths, and surrounding construction. Is it from an independent consultant with a PE stamp?” Requirement: Assembly Thermal Efficiency ≥ 93%. Chi ≤ 0.005 Btu/(hr·°F) per bracket. 12” simulation boundary. Independent third-party per ISO 10211. Project-specific. Red flag: Product modeled in isolation. No chi-value for brackets. Manufacturer-performed modeling. Generic (not project-specific) report. |

A product must meet ALL SEVEN requirements. If any question goes unanswered, the substitution does not meet specification.

BEFORE YOU ACCEPT: WHAT CHANGES IN YOUR BUILDING?

The substitution is proposed after your energy model was submitted, HVAC was sized, and mechanical was bid. Accepting a lower-performing product may invalidate those downstream systems. Ask:

1.	Has the energy model been re-run with the proposed product’s actual Psi values? Does it still comply with code?

2.	Is the specified HVAC equipment still adequately sized? If not, who pays for the redesign and schedule delay?

3.	Who assumes liability for the increased lifecycle operating costs? (Typical projects: $30,000+/year; $1M+ over 20 years.)

The cost of proving a substitution works almost always exceeds whatever the GC hoped to save.

YOUR COMPLETE DEFENSE TOOLKIT

A2P has prepared a full Specification Defense Guide with:

•  Detailed evaluation criteria for each of the 7 Performance Requirements with building-science rationale

•  Substitution Evaluation Checklist — pass/fail form for your project file

•  Sample rejection language ready to copy into project correspondence

•  Downstream impact assessment — energy code, HVAC, lifecycle cost, condensation risk

•  Project-specific thermal analysis via the GreenGirt XO Building Energy Calculator

Ask your A2P representative for the full guide, or contact us directly:

Advanced Architectural Products (A2P)  |  Manufacturer of a portfolio of continuous insulation and building enclosure systems

(269) 355-1818  •  info@greengirt.com  •  www.GreenGirt.com